Primary Setup Tool (PST) Version V3.2 Adress Setting of SIMATIC NET Industrial Ethernet Products - no functional change opposite version 3.2.. Hide part numberDisplay part numberPart number: 6GK1102-6AA00; ELS TP40. 6GK1102-6AB00; ELS TP40M. 6GK1102-7AA00; ELS TP80. 6GK1105-2AE00; OSM TP22. 6GK1105-3AA00; ESM ITP80. 6GK1105-3AA10; ESM ITP80. 6GK1105-3AB00; ESM TP80. 6GK1105-3AB10; ESM TP80. 6GK1105-3AC00; ESM TP40. 6GK1105-4AA00; OSM BC08. 6GK1411-5AA00; IE/PB LINK. 6GK1411-5AA20; IE/PB Link. 6GK1411-5AB00; IE/PB LINK PN IO. 6GK5204-2BB00-2AA3; SCALANCE X204-2. 6GK5206-1BB00-2AA3; SCALANCE X206-1. 6GK5208-0BA00-2AA3; SCALANCE X208. 6GK5208-0CA00-2AA6; SCALANCE X208PRO. 6GK5408-2FD00-2AA2; SCALANCE X408-2. 6GK5414-3FC00-2AA2; SCALANCE X414-3E. 6GK5744-1ST00-2AA6; SCALANCE W744-1PRO. 6GK5744-1ST00-2AB6; SCALANCE W744-1PRO (USA). 6GK5788-1ST00-2AA6; SCALANCE W788-1PRO. 6GK5788-1ST00-2AB6; SCALANCE W788-1PRO (USA). 6GK5788-2ST00-2AA6; SCALANCE W788-2PRO. 6GK5788-2ST00-2AB6; SCALANCE W788-2PRO (USA). 6GK7343-1BA00-0XE0; COMMUNICATION PROCESSOR CP 343-1. 6GK7343-1CX00-0XE0; COMMUNICATION PROCESSOR CP 343-1 LEAN. 6GK7343-1EX20-0XE0; COMMUNICATION PROCESSOR CP 343-1. 6GK7343-1EX21-0XE0; COMMUNICATION PROCESSOR CP 343-1. 6GK7443-1EX11-0XE0; COMMUNICATION PROCESSOR CP 443-1. 6GK7443-1EX40-0XE0; COMMUNICATION PROCESSOR CP 443-1 ADV.... The Primary Setup Tool V3.2 (PST) is now available for downloading. DESCRIPTION: Primary Setup Tool (PST) The Primary Setup Tool (PST) can be used to allocate an address (e.g. IP address) to SIMATIC NET network components, Ethernet CPs and gateways. The precondition is that these SIMATIC NET devices have a preset ETHERNET (MAC) address and can be reached online in the network. Remark: These settings can also be performed with STEP 7 (see STEP7 documentation). In the future it will be possible to set the addresses of more SIMATIC NET Ethernet products via the PST Tool and STEP 7. Please read the relevant Product Information bulletins on this topic. Module list The address of the following modules can be set via PST Tool. SIMATIC NET CPs: Type; Order No.; More Infos. CP343-1; 6GK7343-1GX20-0XE0; entry ID 15392964. 6GK7343-1EX20-0XE0. CP343-1 Lean; 6GK7343-1CX00-0XE0. CP443-1; 6GK7443-1EX11-0XE0. CP443-1 Advanced; 6GK7443-1EX40-0XE0. Note: "Set IP adress by different means" must be set as source for the modules in the features dialog of the STEP7 Hardware Configuration Tool. If connections (S7, S5 compatible connections, etc.) have been configured via the CPs by means of the STEP7 NETPRO tool, these connections will become invalid in the case of a change of IP address with the PST tool. SIMATIC NET INC (Industrial Network Components) The following SIMATIC NET switches permit address setting by means of the PST tool. Type; Order No.; More Infos. OSM / ESM; Function is supported as from firmware version 2.3.0, see entry ID 14198951 "OSM / ESM Firmware update version 2.3.0" dated 09.01.2003. (also possible with PST V2.0 , see entry ID 14929629. ELS TP40 M; 6GK1102-6AB00; see entry ID14040243 (also possible with PST V2.0 , see entry ID 14929629 möglich !. SCALANCE W788-1PRO; 6GK5788-1ST00-2AA6 6GK5788-1ST00-2AB6; See Product Information or Manual for more details.. SCALANCE W788-2PRO; 6GK5788-2ST00-2AA6 6GK5788-2ST00-2AB6; See Product Information or Manual for more details.. SCALANCE W744-1PRO (Ethernet Client Module); 6GK5744-1ST00-2AA6 6GK5744-1ST00-2AB6; See Product Information or Manual for more details.. SCALANCE X200; to follow; See Product Information or Manual for more details.. SCALANCE X400; to follow; See Product Information or Manual for more details.. Note: The PST Tool only supports SIMATIC NET Ethernet network components with management functionality (Web-Based Management and/or SNMP). Here, the additional possibility exists to call the WEB-Based Management for diagnostics and configuration. SIMATIC NET gateways The following SIMATIC NET gateways permit address setting by means of the PST tool . Type; Order No.; More Infos. IE/PB Link; 6GK1411-5AA00 6GK1411-5AA20; see entry ID 13472319 "Delivery release: Firmware V1.3.23 for SIMATIC NET IE/PB Link (6GK1411-5AA00)". IE/PB-Link PN IO; 6GK1411-5AB00; entry ID 19310075. Note: Link Modules additionally permit setting of PROFIBUS parameters (address, profile, Baud rate). INSTALLATION Please ensure that no older version of the Primary Setup Tool is installed on your PC. If necessary, uninstall the older version. Please proceed as follows: Double click the file setup.exe or select the menu command File -> Execute and enter the path to setup.exe. The required files will then be copied to the selected directory. The DCP (Discovery and Basic Configuration Protocol) will be installed. The DLC (Data Link Control) protocol must be installed subsequently. Supported operating systems The Primary Setup Tool can be installed and executed under the following operating systems: -Windows 2000 Professional SP2 -Windows XP Professional Note: Windows NT is only supported with PST V2.0 , see entry ID 14929629 The installation program is a self-extracting ZIP archive. Double click the file in the Windows Explorer or execute the program via the Windows menu command Start -> Execute. The instructions for further installation are contained in the readme.rtf. PST-Tool_V3.2.zip ( 8396 KB ) DLC Protocol (Windows XP Professional) The DLC protocol must be installed separately for Windows XP, because it is not included in the scope of supply of the operating system. The necessary files are located in the sub-directory "DLC Protocol" in the PST installation directory. Please proceed as follows: -First start with the Windows command Start -> Settings -> Control Panel -Open the icon "Network connections"; -From the list of network cards select the one for which the DLC protocol is to be installed; -Open "Features" with the righthand mouse button; -Press the "Installation" button and select "Protocol"; -Press the "Add to" button and select "Data Carrier"; -Enter the path for the sub-directory "DLC Protocol" in your Primary Setup Tool installation folder and click "OK"; -Confirm the selection of the DLC protocol with OK; The uninstallation is performed via Start-> Settings -> Control Panel -> Software-> Primary Setup Tool. The DCP protocol will also be uninstalled in this case. The DCP protocol will not be uninstalled together with the uninstallation of the PST. To remove the DCP protocol, it must be uninstalled separately. For this purpose you should open the features page (as for installation), select "DCP protocol" and press the "Uninstall" button. More information More information on the Primary Setup Tool (installation, configuration, handling, etc.) is available from the SCALANCE X/W configuration manuals, or from the manual "SIMATIC NET Industrial Ethernet Electrical Lean Switch", chapter 6.2.2 (entry ID: 13471140)... Entry ID:19440762. Date:2006-06-13; Formularbeginn Formularende.... I regard this article....; as helpful; as not helpful...

Copy link; Su.